# bio_conductor.py

import time, board, busio, rtmidi, requests

import adafruit_ads1x15.ads1115 as ADS

from adafruit_ads1x15.analog_in import AnalogIn

# --- CONFIG ---

API_KEY = "YOUR_OWM_API_KEY"

CITY = "Los_Angeles"

THRESHOLD = 5000

# --- INITIALIZE ---

i2c = busio.I2C(board.SCL, board.SDA)

ads = ADS.ADS1115(i2c)

plant = AnalogIn(ads, ADS.P0)

pot = AnalogIn(ads, ADS.P1)

midiout = rtmidi.MidiOut()

midiout.open_virtual_port("PlantStation")

def get_vibe():

try:

r = requests.get(f"http://api.openweathermap.org/data/2.5/weather?q={CITY}&appid={API_KEY}").json()

main = r['weather'][0]['main']

return 127 if "Rain" in main else 64 if "Cloud" in main else 0

except: return 0

last_weather = 0

last_p_val = 0

print("Pi 5 Bio-Station Active...")

while True:

# Weather Sync

if time.time() - last_weather > 1800:

midiout.send_message([0xB0, 40, get_vibe()])

last_weather = time.time()

# Bio-Logic

p_val = plant.value

p_delta = abs(p_val - last_p_val)

sens = pot.value / 65535

if p_delta > (THRESHOLD * (1.1 - sens)):

choice = int((p_val / 65535) * 15)

# Send pattern choices to PD

midiout.send_message([0xB0, 51, choice])

midiout.send_message([0xB0, 52, (choice + 4) % 16])

last_p_val = p_val

time.sleep(0.05)