import time, board, busio, rtmidi, requests

import adafruit_ads1x15.ads1115 as ADS

from adafruit_ads1x15.analog_in import AnalogIn

# --- CONFIG ---

API_KEY = "YOUR_OWM_API_KEY"

CITY = "Los_Angeles"

THRESHOLD = 5000

SMOOTHING_FACTOR = 0.2  # 0.1 is heavy smoothing, 0.9 is raw/fast

# --- INITIALIZE ---

i2c = busio.I2C(board.SCL, board.SDA)

ads = ADS.ADS1115(i2c)

plant = AnalogIn(ads, ADS.P0)

pot = AnalogIn(ads, ADS.P1)

midiout = rtmidi.MidiOut()

midiout.open_virtual_port("PlantStation")

def get_vibe():

try:

r = requests.get(f"http://api.openweathermap.org/data/2.5/weather?q={CITY}&appid={API_KEY}").json()

return 127 if "Rain" in r['weather'][0]['main'] else 64 if "Cloud" in r['weather'][0]['main'] else 0

except: return 0

last_weather = 0

last_p_val = 0

smoothed_val = 0

while True:

# 1. Weather Update

if time.time() - last_weather > 1800:

midiout.send_message([0xB0, 40, get_vibe()])

last_weather = time.time()

# 2. Advanced Smoothing (EMA) Logic

raw_val = plant.value

# This replaces the smooth() function from your uploaded file

smoothed_val = (SMOOTHING_FACTOR * raw_val) + ((1 - SMOOTHING_FACTOR) * smoothed_val)

# 3. Decision Engine

p_delta = abs(smoothed_val - last_p_val)

sens = pot.value / 65535

if p_delta > (THRESHOLD * (1.1 - sens)):

choice = int((smoothed_val / 65535) * 15)

midiout.send_message([0xB0, 51, choice]) # Change Lead Pattern

midiout.send_message([0xB0, 52, (choice + 4) % 16]) # Change Pad Pattern

last_p_val = smoothed_val

time.sleep(0.05)