EMS: epiphytic music system 

Initial Documentation

TOC:

0. PROJECT GOALS/TLDR

1. PLANT SENSOR

2. PI Sensor Code and Synth
3. Camera Setup and Stream

====

0.0. PROJECT GOALS
GUGN: Global urban Garden Network - Helping turn grey into green
EMS KIT: Helping urban gardens thrive through music and connection

What is the EMS?

1. Listen to plants and watch them thrive: THE EMS is an Open Source plant music video streaming kit for plants of all types, everywhere.

2. A standalone Raspberry pi based kit for plant signal, synth, video streaming, so you can stream your garden as it helps itself thrive. 

How:.

MVP EMS v1:

Raspberry Pi, Plant sensor, Pi camera, audio and video outputs.
.

So basically just need signal amplified and converted, then the rest is inside PD with Python (not that I have done that inside PD, but hey, next steps).

2 super cheap chips into Pi. Python electric to midi, with plant specific Midi filters in PD, PD synth, audio out to youtube streaming, audio out to speaker.

Camera to Pi, mux audio in pi, stream to youtube.  Starting documentation below

TLDR:

Answer, super simple PI implementation of sensor, with a couple of amp chips and a Pi camera to stream

LISTEN: PLANT —>Alligator/EEG Pads—> Sensor Hat/board—>Pi—>Python filter→PD Vanilla Synth and Sequencer—>Audio Out, TRS Midi Out, (if possible BTLE audio out)—>Speaker (can be part of box or separate)

WATCH: PI—>Audio From Plant→*Possibly audio from microphone*-->Camera video→MUX ffmpeg—>Stream to Youtube

Code start, schematics

----

So here's midisprout that we currently use:

https://github.com/electricityforprogress/MIDIsprout

It uses a different ADC, and might be easier?

he made a more sophisticated one with BLE etc. but it is discontinued

https://github.com/electricityforprogress/BiodataFeather

instructable

https://www.instructables.com/Biodata-Sonification/

--------------->>> RASPBERRY PI SOLUTION<----------------

2 ideas:

1. Use Raspberry pi for plant signal

2. Create standalone plug and play camera system (aka, 2 products/modes, 1 for video streaming and 1 for just plant music).

Solution:

Raspberry Pi, 2 chips, Pi camera and extensions. See below...

So basically just need signal amplified and converted, then the rest is inside PD with Python (not that I have done that inside PD, but hey, next steps).

Hardware SENSITIVE VERSION:

INA 333  ​​The device can be operated by a power supply as low as 1.8V (± 0.9V) with a quiescent current of only 50 μA,

- while also providing ultra-low noise density (50nV/√Hz) that extends down to DC.

- Low input bias current: 200pA (maximum) power supply

- Input voltage:(V-) +0.1V to (V+ ) -0.1V

- Output voltage:(V-) +0.05V to (V+ ) -0.05V

Low quiescent current: 50 μA operating temperature range:-40°C to 125°C Filtered RF interference (RFI) input

ADS 1115 https://www.adafruit.com/product/1085#tutorials%5D

HARDWARE OLDSCHOOL CAPACITIVE
 (555 Timer Version)

- Raspberry Pi 5:

- NE555 Timer IC: The heart of the frequency generator.

- Resistors: $10\text{k}\Omega$ fixed and a $100\text{k}\Omega$ Potentiometer (for calibration).

- Capacitor: $0.1\mu\text{F}$ ceramic capacitor.

- USB Stereo Audio Interface: For high-quality nature sound input and master output to the stream.

- 2x MAX4466 Mics: For stereo nature ambience. 

PI CAMERA https://www.adafruit.com/product/5660

Camera LED DORHEA 2 Pcs Infrared LED Light Board Module for Raspberry Pi Camera Night Vision, 3W 850 IR High Power Night Vision Infrared Illuminator with Adjustable Resistor for Raspberry Pi Camera

PI Vilros Raspberry Pi 5-8GB Starter Kit - Turbo Cooled Edition - 32GB Memory (Aluminum Black)

ADDIDITIONAL HARDWARE AS BACKUP

XYGStudy RTC Battery for... https://www.amazon.com/dp/B0CR76SM52?ref=ppx_pop_mob_ap_share

Other ADS1115 CQRobot ADS1115 16-Bit ADC Conversion Module Compatible with Raspberry Pi and Arduino Boards. Achieve Analog and Digital Signal Acquisition or Conversion, 3.3V to 5V, I2C Address Selection Switch.

Other ADS 1115 Teyleten Robot ADS1115 16 Bits 4 Channel Analog-to-Digital Converter Precised Develop Board Module Amplifier Board ADC I2C IIC for Arduino Raspberry Pi (Pack of 3pcs)

LIKELY DOESN”T FIT Arducam for Raspberry Pi Camera Module 3 Case, ABS Housing for IMX519 16MP Autofocus Camera, Compatible with Raspberry Pi Camera Module 3/V1/V2, and Any 25 * 24mm Camera Board

SENSOR OPTIONS.

Option A)
333 amp to ADS

Option B) Midi Sprout Technique

Option C) No INA333

OPTION A)
INA 333 Sensor to ADS1115

Parts list

- 2 × 100k (VBias divider)

- 1 × 4.7µF cap (VBias stability)

- 2 × 10M (bias each input to VBias)

- 2 × 100k (series “lead isolators”)

- 2 × 1nF (to VBias, matched)

- Optional: 10nF across IN+ and IN-

- 1 × RG (11k to start)

Key idea: on a single supply, you bias everything to mid-supply and use INA333’s REF pin so the output sits around mid-supply when the leaf differential is ~0. (Texas Instruments)

## Easiest wiring (INA333 → ADS1115)

### 1) Power + mid-supply reference (VBias ≈ 1.65V)

3.3V ---[100k]---+---[100k]--- GND

|

VBias  (~1.65V)

|

[4.7µF]

|

GND

- 
INA333 V+ = 3.3V, V- = GND

- INA333 REF = VBias (this is important)

INA333 works on low single supplies (down to 1.8V) and uses the standard gain equation below. (Texas Instruments)

### 2) Leaf clips + bias + cable-noise filtering (recommended for 3 ft)

Put the resistors/caps near the INA333, not at the leaf.

Clip1 ----(3ft)----[100k]----+----> INA333 IN+

|

[10M]

|

VBias

|

[1nF]

|

VBias

Clip2 ----(3ft)----[100k]----+----> INA333 IN-

|

[10M]

|

VBias

|

[1nF]

|

VBias

Optional (often helps a lot):

INA333 IN+ ----||---- INA333 IN-     (10nF)



### 3) Set gain with one resistor (RG)

INA333 gain: G = 1 + (100kΩ / RG). (Texas Instruments)

Good starter gains (so you don’t instantly clip):

- G ≈ 5 → RG ≈ 25k (use 24.9k)

- G ≈ 10 → RG ≈ 11.1k (use 11k) ✅ my usual starting point

- G ≈ 20 → RG ≈ 5.26k (use 5.23k)

If your output hits 0V or 3.3V often, reduce gain (increase RG).

### 4) Output to ADS1115

INA333 OUT  ----> ADS1115 A0 (single-ended)

ADS1115 GND ----> same GND as INA333/Pi

ADS1115 VDD ----> 3.3V

Configure ADS1115 to read A0 (single-ended) and keep a modest PGA range (start at ±2.048V).

### Interference Notes:

Here’s how I’d tune the INA333 + ADS1115 setup to behave on wall power (still “easy,” just a few value changes + wiring habits).

## Do these 6 things

### 1) Use twisted pair (or shielded twisted pair) for the 3 ft lead

- Best: shielded twisted pair (mic cable / instrument cable style).

- Connect the shield to GND at the electronics end only (leave shield floating at the leaf).

### 2) Make VBias lower-impedance (more stable)

Instead of 100k/100k, use 10k/10k and more capacitance:

- 3.3V → 10k → VBias → 10k → GND

- VBias to GND: 10 µF + 0.1 µF (both)

This makes VBias stiffer so your bias network doesn’t “wobble” with noise.

### 3) Keep the input bias resistors, add stronger input filtering (still symmetric)

At the INA333 inputs (right at the pins):

- Each input still gets 10 MΩ to VBias

- Each lead still gets a series resistor: keep 100k (good for “cable isolation”)

Now bump the caps:

- IN+ to VBias: 4.7 nF

- IN− to VBias: 4.7 nF

- IN+ to IN− (differential cap): 47 nF

This combo usually knocks down 60 Hz a lot with 3 ft leads, while still leaving plenty of “plant motion” responsiveness.

### 4) Add proper decoupling at the INA333

Right at INA333 power pins:

- 0.1 µF from 3.3V to GND

- plus 1–10 µF from 3.3V to GND

(Physically close matters more than value.)

### 5) Add a gentle low-pass on the INA output before ADS1115

This cleans up residual buzz before it hits the ADC:

- INA333 OUT → 1k series → ADS1115 A0

- ADS1115 A0 → 1 µF to GND

That gives a simple low-pass that’s still responsive for plant signals.

### 6) Reduce ground-loop opportunities

- Plug everything (Pi + any peripherals) into the same power strip.

- If you’re using other audio gear / computers: that’s where loops happen. When possible, keep the sensor electronics on the Pi only.

- Put a ferrite clamp on the Pi’s power cable (cheap, often helps)

## Gain setting (keep it conservative on wall power)

Start with G ≈ 5 to 10:

- G ≈ 10 → RG = 11k

- If it clips or “rails” often, drop to G ≈ 5 → RG = 24.9k

## ADS1115 settings that help on wall power

- Sample slow: 8–32 SPS

- Then in software, do a moving average (e.g., average the last 8–32 samples)

🔌 ADS1115 – 16-bit ADC Breakout Boards:

- PREFERRED: Adafruit ADS1115 16-Bit ADC - 4 Channel with Programmable Gain Amplifier

  - Price: $14.95

  - Features: 4 single-ended or 2 differential inputs, programmable gain amplifier, I²C interface.

  - Link: Adafruit Product Page

BACKUPS

- ShillehTek ADS1115 Pre-Soldered Analog to Digital Converter

  - Price: $8.98

  - Features: Pre-soldered for easy setup, compatible with Raspberry Pi and Arduino.

  - Link: ShillehTek Product Page

- Comidox 2PCS 16 Bit 4 Channel I2C ADS1115 Module

  - Price: Varies

  - Features: Comes in a pack of 2, suitable for multiple projects.

  - Link: Amazon Product Page

1. Raspberry Pi (4 or 5)

- Already has I²C pins and audio/MIDI output capabilities.

- Recommended: Raspberry Pi 4 or 5 with Raspbian (or Raspberry Pi OS Lite

Connect ADS1115 to Sensors and Pi

You’ll Need:

- Raspberry Pi (any model with GPIO, e.g. Pi 4 or 5)

- ADS1115 breakout board (e.g., Adafruit or Waveshare)

- Jumper wires and breadboard

- Two electrodes (metal clips, ECG pads, or copper tape)

1. Wiring: ADS1115 to Raspberry Pi

ADS1115 uses I²C. Connect as follows:

| ADS1115 Pin | Connect To (Pi GPIO) |
| --- | --- |
| VDD | 3.3V (Pin 1) |
| GND | GND (Pin 6) |
| SDA | SDA (GPIO2 / Pin 3) |
| SCL | SCL (GPIO3 / Pin 5) |
| ADDR | GND (for 0x48 address) |
| A0 | From sensor/amplifier |

NOTE: If you’re not using an amplifier, you can connect the sensor directly to A0 (AIN0), but it must be within 0–3.3V.

2. Plant Sensor Wiring

If no INA126: connect electrodes across A0 and GND of ADS1115 directly. But signals will be weak.

3. Enable I²C on Pi

bash

CopyEdit

sudo raspi-config

# Go to "Interfacing Options" → "I2C" → Enable

sudo reboot

Install dependencies:

bash

CopyEdit

sudo apt update

sudo apt install python3-pip i2c-tools

pip3 install adafruit-circuitpython-ads1x15

Test connection:

bash

i2cdetect -y 1

# You should see `0x48` or similar address appear

4. Python Code to Read the Sensor

python

CopyEdit

import time

import board

import busio

import adafruit_ads1x15.ads1115 as ADS

from adafruit_ads1x15.analog_in import AnalogIn

# Create I2C bus

i2c = busio.I2C(board.SCL, board.SDA)

# Create ADC object

ads = ADS.ADS1115(i2c)

# Create single-ended input on channel 0

chan = AnalogIn(ads, ADS.P0)

# Continuously read and print

while True:

print(f"Raw: {chan.value}, Voltage: {chan.voltage:.5f} V")

time.sleep(0.2)

This will give you voltage readings from your plant sensor, suitable for MIDI or sound triggers.

🧰 Assembly Tips

- ADS1115 Connection: Connect the ADS1115 to your Raspberry Pi via the I²C interface (SDA and SCL pins). Ensure you enable I²C on your Raspberry Pi.

II. PD/Synth

III. CAMERA

1. Camera Module Options for Raspberry Pi

Recommended Camera:

- Raspberry Pi Camera Module 3 (Wide or NoIR version)

  - Sony IMX708 sensor (great low-light performance)

  - Available with HDR, autofocus, and NoIR (infrared-sensitive for night)

  - Uses CSI ribbon cable interface (not USB)

- Buy from Raspberry Pi Store

2. Waterproofing and Outdoor Enclosures

Options:

- IP66/67 Outdoor Camera Housing for Pi Camera

  - Weatherproof enclosures specifically designed for Pi Camera modules

  - Often include slots for the ribbon cable and clear acrylic or glass lens cover

  - Examples: Arducam outdoor housing, or generic IP67 project boxes with DIY mounting

- Extendable Ribbon Cable (CSI FFC)

  - Available up to 1–2 meters, but signal quality degrades beyond ~1 meter without boosters

  - Arducam FPC Extension Kit
boosts signal to 10+ meters using differential pair conversion

3. Power + 24/7 Reliability Considerations

- Cooling: Enclosure should allow some ventilation or include heatsinks, especially in sun

- Power: Use a PoE HAT or high-quality USB-C 5V 3A supply

- Thermal Monitoring: Optional scripts to reboot or alert on thermal events

- Storage: Use high-endurance SD card (or external SSD via USB)

4. Streaming 24/7 with Camera

You can use:

- FFmpeg to stream to YouTube or Twitch

- GStreamer or Motion for RTSP

- UV4L or libcamera-vid for local/low-latency streaming

Camera functionality is built in

https://www.raspberrypi.com/documentation/computers/camera_software.html

CODE:

***There are a few versions of code I have

Here, I am adding some code to get weather data as well. An option, not hard usually, but we can decide if we want to factor it. ****

Hardware debug:
Plant Diagnostic Code

Unsmoothed python via Gemini
# bio_conductor python

Smoothed
Plant Python gemini 2



GPT python Smoothed:
# GPT plant code smoothed

For 555 Timer Version
#-- 555 python Timer code—

PD code with gui via bop and variability
PD code with variability and gui

PD code simpler
PD patches using bop

Camera Code:
FFMPEG Camera To youtube

Assembly Tip:

For camera ribbon cables exiting the case:

- Use flat FPC gaskets or Arducam FPC extender kits (they convert CSI to Ethernet or shielded cabling for long runs)

II. 

SECOND VERSION, WITH MORE PD