INA 333 option

Here’s the easiest “works on a Raspberry Pi” INA333 setup for your case (2 clips on 1 leaf, ~3 ft leads) feeding an ADS1115.

Key idea: on a single supply, you bias everything to mid-supply and use INA333’s REF pin so the output sits around mid-supply when the leaf differential is ~0. (Texas Instruments)

## Easiest wiring (INA333 → ADS1115)

### 1) Power + mid-supply reference (VBias ≈ 1.65V)

3.3V ---[100k]---+---[100k]--- GND

|

VBias  (~1.65V)

|

[4.7µF]

|

GND

- 
INA333 V+ = 3.3V, V- = GND

- INA333 REF = VBias (this is important)

INA333 works on low single supplies (down to 1.8V) and uses the standard gain equation below. (Texas Instruments)

### 2) Leaf clips + bias + cable-noise filtering (recommended for 3 ft)

Put the resistors/caps near the INA333, not at the leaf.

Clip1 ----(3ft)----[100k]----+----> INA333 IN+

|

[10M]

|

VBias

|

[1nF]

|

VBias

Clip2 ----(3ft)----[100k]----+----> INA333 IN-

|

[10M]

|

VBias

|

[1nF]

|

VBias

Optional (often helps a lot):

INA333 IN+ ----||---- INA333 IN-     (10nF)



### 3) Set gain with one resistor (RG)

INA333 gain: G = 1 + (100kΩ / RG). (Texas Instruments)

Good starter gains (so you don’t instantly clip):

- G ≈ 5 → RG ≈ 25k (use 24.9k)

- G ≈ 10 → RG ≈ 11.1k (use 11k) ✅ my usual starting point

- G ≈ 20 → RG ≈ 5.26k (use 5.23k)

If your output hits 0V or 3.3V often, reduce gain (increase RG).

### 4) Output to ADS1115

INA333 OUT  ----> ADS1115 A0 (single-ended)

ADS1115 GND ----> same GND as INA333/Pi

ADS1115 VDD ----> 3.3V

Configure ADS1115 to read A0 (single-ended) and keep a modest PGA range (start at ±2.048V).

## Parts list (easy to source)

- 2 × 100k (VBias divider)

- 1 × 4.7µF cap (VBias stability)

- 2 × 10M (bias each input to VBias)

- 2 × 100k (series “lead isolators”)

- 2 × 1nF (to VBias, matched)

- Optional: 10nF across IN+ and IN-

- 1 × RG (11k to start)

### One practical note

Even with an INA, 3 ft leads will still pick up hum unless the two leads are a twisted pair (or shielded twisted pair with shield grounded only at the electronics end).

If you tell me whether you’re powering everything from battery vs wall power, I’ll suggest the best cap values (battery can usually run less filtering; wall power often needs more).

Wall power is the “hard mode” for this, because you’ll almost always pick up 60 Hz hum + switching-supply noise through the 3 ft leads and through ground.

Here’s how I’d tune the INA333 + ADS1115 setup to behave on wall power (still “easy,” just a few value changes + wiring habits).

## Do these 6 things

### 1) Use twisted pair (or shielded twisted pair) for the 3 ft lead

- Best: shielded twisted pair (mic cable / instrument cable style).

- Connect the shield to GND at the electronics end only (leave shield floating at the leaf).

### 2) Make VBias lower-impedance (more stable)

Instead of 100k/100k, use 10k/10k and more capacitance:

- 3.3V → 10k → VBias → 10k → GND

- VBias to GND: 10 µF + 0.1 µF (both)

This makes VBias stiffer so your bias network doesn’t “wobble” with noise.

### 3) Keep the input bias resistors, add stronger input filtering (still symmetric)

At the INA333 inputs (right at the pins):

- Each input still gets 10 MΩ to VBias

- Each lead still gets a series resistor: keep 100k (good for “cable isolation”)

Now bump the caps:

- IN+ to VBias: 4.7 nF

- IN− to VBias: 4.7 nF

- IN+ to IN− (differential cap): 47 nF

This combo usually knocks down 60 Hz a lot with 3 ft leads, while still leaving plenty of “plant motion” responsiveness.

### 4) Add proper decoupling at the INA333

Right at INA333 power pins:

- 0.1 µF from 3.3V to GND

- plus 1–10 µF from 3.3V to GND

(Physically close matters more than value.)

### 5) Add a gentle low-pass on the INA output before ADS1115

This cleans up residual buzz before it hits the ADC:

- INA333 OUT → 1k series → ADS1115 A0

- ADS1115 A0 → 1 µF to GND

That gives a simple low-pass that’s still responsive for plant signals.

### 6) Reduce ground-loop opportunities

- Plug everything (Pi + any peripherals) into the same power strip.

- If you’re using other audio gear / computers: that’s where loops happen. When possible, keep the sensor electronics on the Pi only.

- Put a ferrite clamp on the Pi’s power cable (cheap, often helps).

## Gain setting (keep it conservative on wall power)

Start with G ≈ 5 to 10:

- G ≈ 10 → RG = 11k

- If it clips or “rails” often, drop to G ≈ 5 → RG = 24.9k

## ADS1115 settings that help on wall power

- Sample slow: 8–32 SPS

- Then in software, do a moving average (e.g., average the last 8–32 samples)

That combo beats chasing noise with more analog gain.

If you want, tell me: is the leaf clip setup near lights / a laptop charger / a monitor? Those are the top 3 hum injectors, and I can suggest the single most effective placement change.