# EMS - Notes

# Overview (how it works)

- Using Galvanic Conductance - skin conductance -

- changes in electrical conductivity are measured

- Changes are detected using averages and standard deviation calculations

- circuit used to detect biological galvanic conductance is based on a 555 timer IC set as an astable multivibrator

## Engineering

- EDN article about how

- essentially pulsing known voltage and measuring current

- Conductace G = 1/R = I / V

- using the 555, you vary the frequency based on the resistance

- do we measure frequency? or voltage?

- TI forum post about using instrumentation amplifier for measuring signal

  - report using INA826 to measure

# Hardware

- Computer

  - RPi5

    - what OS should i use?

    - I need a monitor and keyboard.. maybe i can configure it headless and SSH into it

- Amplifiers

  - INA333 module - QTY 2 - instrumentation amplifier - link - INA333 Datasheet

    - Module schematic!

  - AD620 module -QTY 4- instrumentation amplifier -  amazon link

  - INA129 - QTY 1 -  instrumentation amplifier - amazon link

- ADC - Analog to Digital converts

  - ADS1115 - 16 bit i2c adc - adafruit

    - pinouts from adafruit

    - I2C address is 0x48

    - A+ and A- - ADC power supply (VIN through a ferrite) and ADC ground (digital GND through a ferrite) these are OUTPUTs not inputs!

  - ADS1115 - Amazon

  - ADS 1115 module - CQrobot - QTY 1 - amazon link - has flying leads

- Camera

  - FPC cables - for Camera - 500mm, 200mm

  - QTY 2 - Rpi Camera Module 3

- Photocell + light or ultrasonic sensor module… QTY 2

- CMOS battery - QTY 1

- Microphone - QTY 2 -

- electrode pads - ECG electrodes - QTY 4

Two methods

- amplifier + ADC

  - INA333 - rail to rail instrumentation amplifier - datasheet

  - ADS1115 - 16 bit ADC - datasheet

- 555 timer - measure frequency?

# Approach

## Parts of Project

- Getting Analog Signal from plant

  - either frequency from 555

  - or voltage output of instrumentation amplifier

- Converting analog signal to digital

- reading signal into RPI

- converting signal into something to send to PD

- PD music making

- PD output via headphone jack

- PD output to stream

- Camera INPUT to RPI

- Camera streaming via wifi

## Parts (summarized)

- analog plant signal

- RPI setup to read signal and convert to PD

- Camera INPUT and streaming

# Code

- Python code

  - brings in data from ADC

  - smoothes / filters it

  - convert voltage to midi

- PD Code

- Bop Library For PD

# Project Requirements

- signal from plant

- music from PD from plant

- camera signal sent to live stream

## Nice to haves

- pot to control sensitivity

- led to show notes playing

## Needs

- monitor, keyboard for RPI, (SD card reader?) probably have one

- PLANT

# Progress

Jan 30, 2026

- Installing PatchBox OS

- No monitor so want to SSHinto it. cant find IP address and can’t connect to its access point..

- ended up connected via phone and the patchbox wifi network

  - password: blokaslabs

- ssh patch@172.24.1.1 works on phone

- Over ethernet ping patchbox.local and i got ip6 address of fe80::6e33:2b76:405:55ab and that is confirmed by going into phone ssh.

  - for some reason not getting ip4 address.

- ssh patch@fe80::6e33:2b76:405:55ab should work via ethernet, but its not working

- only works via phone. Need monitor and keyboard. will need mouse too eventually to use PD Gui

- NEXT TIME: get plant, screen, keyboard, mouse

Feb 6, 2026

- meeting with Daniel

- got patchbox running and PD running with mouse, keyboard, screen

- Goals

  - get plant data into raspberry pi

  - get audio out and listening through speakers

- Electrode Pinout on ⅛” TRS jack

  - LEFT (yellow) = top pin (jack) = tip (plug)

  - RIGHT (red) = bottom pin (jack) = sleeve (plug)

  - ring is unused

- Wiring up INA333 and ADS1115

  - A+ and A- - ADC power supply (VIN through a ferrite) and ADC ground (digital GND through a ferrite) these are OUTPUTs not inputs!

  - USE these if signal is too noisey? compare and contrast.

LINKAGE
https://zeal.co/projects/bop/
https://github.com/zealtv/bop/releases