To consolidate your project, we will bridge the high-precision biodata logic of the MIDI Sprout (original hardware logic) with the modern, modular synthesis of the BOP (zealtv) library.

### 1. Consolidated Schematic (High-Level)

Since I am a text-based AI, I have generated a visual logic diagram below that represents the consolidated schematic. You can use this as your wiring guide.

Plaintext



[ PLANT LEAVES ]

|

[ ELECTRODES ]  <-- MIDI Sprout Style (Conductivity)

|

+-------v-------+

|   INA333      | (Instrumental Amp for

|  Bio-Potent.  |  Micro-fluctuation gain)

+-------+-------+

|

+-------v-------+        [ 10k Pot ]   [ MAX4466 ]

|    ADS1115    <----------+ (Sens)      + (Mic)

| (I2C ADC)     |

+-------+-------+

| (SDA/SCL)

+-------v-------+

| Raspberry Pi  | --- [ USB/BT Speaker ]

| (Python + PD) | --- [ YouTube Stream ]

+---------------+



### 2. Consolidated Python Script (plant_bop.py)

This script implements the MIDI Sprout logic (calculating the timing between fluctuations) but sends it as MIDI CC and Notes directly into the virtual MIDI port for BOP to handle.

Python



import board

import busio

import adafruit_ads1x15.ads1115 as ADS

from adafruit_ads1x15.analog_in import AnalogIn

import rtmidi

import time

# MIDI Setup

midiout = rtmidi.MidiOut()

midiout.open_virtual_port("BOP_Plant")

# Hardware Setup

i2c = busio.I2C(board.SCL, board.SDA)

ads = ADS.ADS1115(i2c)

plant_sensor = AnalogIn(ads, ADS.P0)

pot = AnalogIn(ads, ADS.P1)

# MIDI Sprout Logic Variables

last_val = 0

threshold = 1000  # Sensitivity floor

print("BOP Plant System Active...")

try:

while True:

raw_val = plant_sensor.value

sensitivity = pot.value / 65535 # 0.0 to 1.0

# Calculate Delta (The "Sprout" logic: change matters more than value)

diff = abs(raw_val - last_val)

if diff > (threshold * (1 - sensitivity)):

# Map diff to a MIDI Note (Scale clipping happens in PD)

note = int((raw_val / 65535) * 127)

velocity = min(127, int((diff / 5000) * 127))

# Send Note On

midiout.send_message([0x90, note, velocity])

time.sleep(0.05) # Note duration

midiout.send_message([0x80, note, 0])

# Send Continuous Data for BOP Filters

midiout.send_message([0xB0, 1, int(sensitivity * 127)]) # CC 1 for BOP Matrix

last_val = raw_val

time.sleep(0.01)

except KeyboardInterrupt:

midiout.close_port()



### 3. Pure Data Patch (The "BOP" Matrix)

To use the bop library, ensure you have cloned the repo into your PD path. Below is the text-representation of your patch. It uses BOP's modular synthesis and a Matrix Filter to guide the plant's notes.

Save as plant_stream.pd:

Plaintext



#N canvas 500 200 600 600 12;

#X obj 50 50 notein;

#X obj 50 80 bop.semisteps;  <-- BOP scale quantizer (C-Minor Pent)

#X msg 120 80 0 3 5 7 10;    <-- Define the scale matrix here

#X obj 50 120 bop.synth~;    <-- The BOP Sound Engine

#X obj 200 120 ctlin 1;      <-- Sensitivity Pot (CC 1)

#X obj 50 200 bop.reverb~;   <-- BOP Ambient effects

#X obj 50 250 bop.delay~;

#X obj 300 100 bop.ui;       <-- Launch the BOP GUI to tweak visuals

#X obj 50 350 dac~;          <-- Output to Audio/Stream

#X obj 50 320 *~ 0.5;        <-- Master Volume

#X connect 0 0 1 0;          <-- Raw note to scale quantizer

#X connect 1 0 3 0;          <-- Quantized note to BOP synth

#X connect 2 0 1 1;          <-- Send scale matrix to quantizer

#X connect 4 0 3 1;          <-- Map Pot to Synth Filter/Cutoff

#X connect 3 0 5 0;          <-- Synth to Reverb

#X connect 5 0 6 0;          <-- Reverb to Delay

#X connect 6 0 9 0;          <-- Signal to Volume

#X connect 9 0 8 0;

#X connect 9 0 8 1;

### Key Logic Additions:

- BOP Matrix Filtering: In the PD code, bop.semisteps acts as your filter. By sending it a message like 0 2 4 7 9 (Major) or 0 3 5 7 10 (Minor), you restrict the plant's chaotic input to a "guided" musical structure.

- Dynamic Cutoff: The Python script sends the Potentiometer value as CC 1. In PD, this is mapped to the bop.synth~ input, allowing you to manually filter the "brightness" of the plant's voice in real-time while it streams.

- BOP UI: Adding [bop.ui] lets you see the parameter states on your Pi's desktop, which is helpful for the YouTube stream's visual component.